General Comments

- Get rough power budgeting per slice (each board takes about 200 mA)

- Double-check footprints

- (Non-critical) Determine why the diodes are on the buck converter

Schematic

- Check footprints and match barrel jacks

- Need more amperage for the entire board (output 1.5 - 2A), make sure we can handle it

- Add another set of bulk capacitors on the 3v3 and 5v

- Tie MFP from the RTC to one of the Nucleo I/O pins, add a decoupling capacitor and pullup resistor to the MFP line

- Use a 36-pin edge connection symbol

- Find out the power draw on the 0-ohms and ensure that it can handle the current draw (0805 appears that it’s only rated for 1/8 W?)

- Slow-blow fuse for lines going out to the motors and actuation slice

- Add 3V3 and 5V inputs bypassing the bucks - need to add bypasses for the bucks (0-ohm jumper)

Board Design

- Move more elements to the back if possible

- Determine the amp limit through the edge connector, make sure that it is also limited with a fuse for safety (3A per contact)

  - Keep this conservative!

  - Put multiple traces for each 5V (?) to power each board

- Size traces and vias

- Add slits under each current sensor

- Use the recommended layout for the RTC

- Tune capacitors for the RTC

- Make the electrolytic capacitors traces thicker